# Introduction

Introduction

This chapter will introduce you to the basics of the Cortex-M4F and the Stellaris peripherals. The lab will step you through setting up the hardware and software required for the rest of the workshop.

The Wiki page for this workshop is located here:

www.ti.com/StellarisLaunchPadWorkshop

## Chapter Topics

Introduction	1-1

Chapter Topics	1-2

TI Processor Portfolio and Stellaris Roadmap	1-3

Stellaris LM4F120 Series Overview	1-4

LM4F120H5QR Specifics	1-5

LaunchPad Board	1-7

Lab1: Hardware and Software Set Up	1-8

Objective	1-8

Procedure	1-9

## TI Processor Portfolio and Stellaris Roadmap

## Stellaris LM4F120 Series Overview

## LM4F120H5QR Specifics

www.ti.com/StellarisPinMuxUtility

## LaunchPad Board

## Lab1: Hardware and Software Set Up

### Objective

The objective of this lab exercise is to download and install Code Composer Studio, as well as download the various other support documents and software to be used with this workshop.  Then we’ll review the contents of the evaluation kit and verify its operation with the pre-loaded quickstart demo program. These development tools will be used throughout the remaining lab exercises in this workshop.

### Procedure

#### Hardware

- You will need the following hardware:

  - A 32 or 64-bit Windows XP or Windows7 laptop with 2G or more of free hard drive space. 1G of RAM should be considered a minimum … more is better.

  - A laptop with Wi-Fi is highly desirable

  - If you are working the labs from home, a second monitor will make the process much easier. If you are attending a live workshop, you are welcome to bring one.

  - If you are attending a live workshop, please bring a set of earphones or ear-buds.

  - If you are attending a live workshop, you will receive an evaluation board; otherwise you need to purchase one. (http://www.ti.com/tool/EK-LM4F120XL)

  - If you are attending a live workshop, a digital multi-meter will be provided; otherwise you need to purchase one like the inexpensive version here:
( http://www.harborfreight.com/catalogsearch/result?q=multimeter )

  - If you are attending a live workshop, you will receive a second A-male to micro-B-male USB cable. Otherwise, you will need to provide your own to complete Lab 7.

  - If you are attending a live workshop, you will receive a Kentec 3.5” TFT LCD Touch Screen BoosterPack (Part# EB-LM4F120-L35).  Otherwise, you will need to provide your own to complete Lab 10.

As you complete each of the following steps, check the box in the title, like the below, to assure that you have done everything in order.

#### Download and Install Code Composer Studio

- Download and start the latest version of Code Composer Studio (CCS) 5.x web installer from http://processors.wiki.ti.com/index.php/Download_CCS (do not download any beta versions). Bear in mind that the web installer will require Internet access until it completes. If the web installer version is unavailable or you can’t get it to work, download, unzip and run the offline version. The offline download will be much larger than the installed size of CCS since it includes all the possible supported hardware. 

This version of the workshop was constructed using build number 5.3.0.00090. Your version will likely be later. For this and the next few steps, you will need a my.TI account (you will be prompted to create one or log into your existing account).

You should note that the 16K limitation on the free, code size limited version of CCS is too small to work with most of the projects in this workshop.

Note that the evaluation license of CCS will operate with full functionality for free while connected to a Stellaris evaluation board. Most Stellaris boards can also operate as an emulator interface for your target system, although this function requires a licensed version of CCS.

- If you have downloaded the offline file, start the ccs_setup_5.xxxxx.exe file in the folder created when you unzipped the download.

- Accept the Software License Agreement and click Next.

- Unless you have a specific reason to install CCS in another location, accept the default installation folder and click Next. If you have an another version of CCS and you want to keep it, we recommend that you install this version into a different folder.

- Select “Custom” for the Setup type and click Next.

- The next dialog, select the processors that your CCS installation will support. You should select “Stellaris Cortex M MCUs” in order to run the labs in this workshop. If you are also attending the MSP430 workshop you should also select “MSP430 Low Power MCUs”. You can select other architectures, but the installation time and size will increase. Click Next.

- In the Component dialog, keep the default selections and click Next.

- In the Emulators dialog, uncheck the Blackhawk and Spectrum Digital emulators, unless you plan on using either of these.

- When you reach the final installation dialog, click Next. The web installer process should take 15 - 30 minutes, depending on the speed of your connection. The offline installation should take 10 to 15 minutes. When the installation is complete, don’t start CCS.

#### Install StellarisWare

- Download and install the latest full version of StellarisWare from: http://www.ti.com/tool/sw-lm3s  . This workshop was built using release number 9107. Your version will likely be a later one. If at all possible, please install StellarisWare into the default C:\StellarisWare folder.

#### Install LM Flash Programmer

- Download, unzip and install the latest LM Flash Programmer (LMFLASHPROGRAMMER) from http://www.ti.com/tool/lmflashprogrammer  .
This workshop was built using version number 1381. Your version will likely be a later one.

#### Download ICDI Drivers

- Download the latest version of the in-circuit debug interface drivers from http://www.ti.com/tool/stellaris_icdi_drivers . Unzip the file and place the stellaris_icdi_drivers folder in C:\StellarisWare.

#### Download and Install Workshop Lab Files

- Download the lab installation file from the workshop materials section of the Wiki site below. The file will install your lab files in: C:\StellarisWare\boards\MyLaunchPadBoard . So please be sure that you have installed StellarisWare before installing the labs.

www.ti.com/StellarisLaunchPadWorkshop

#### Download Workshop Workbook

- Download a copy of the workbook pdf file from the workshop materials section of the Wiki site below to your desktop. It will be handy for copying and pasting code.

www.ti.com/StellarisLaunchPadWorkshop

#### Terminal Program

- If you are running WindowsXP, you can use HyperTerminal as your terminal program. Windows7 does not have a terminal program built-in, but there are many third-party alternatives. The instructions in the labs utilize HyperTerminal and PuTTY. You can download PuTTY from the address below.

http://the.earth.li/~sgtatham/putty/latest/x86/putty.exe

#### Windows-side USB Examples

- Download and install the StellarisWare Windows-side USB examples from this site:

www.ti.com/sw-usb-win

#### Download and Install GIMP

- We will need a graphics manipulation tool capable of handing PNM formatted images. GIMP can do that. Download and install GIMP from here:

www.gimp.org

#### LaunchPad Board Schematic

- For your reference, the schematic is included at the end of this workbook.

#### Helpful Documents and Sites

- There are many helpful documents that you should have, but at a minimum you should have the following documents at your fingertips. 

Look in C:\StellarisWare\docs and find:

Peripheral Driver User’s Guide (SW-DRL-UGxxxx.pdf)

USB Library User’s Guide (SW-USBL-UGxxxx.pdf)

Graphics Library User’s Guide (SW-GRL-UGxxxx.pdf)

LaunchPad Board User’s Guide (SW-EK-LM4F120XL-UG-xxxx.pdf  )

- Go here: http://www.ti.com/product/lm4f120h5qr and download the LM4F120H5QR 
Data Sheet. Stellaris data sheets are actually the complete user’s guide for the device. So expect a large document.

- Download the ARM Optimizing C/C++ Compilers User Guide from http://www.ti.com/lit/pdf/spnu151 (SPNU151). Of particular interest are the sizes for all the different data types in table 6-2. You may see the use of “TMS470” here … that is the TI product number for its ARM devices.

- You will find a “Hints” section at the end of chapter 2. You will find this information handy when you run into problems during the labs.

- Search the TI website for these additional documents of interest:

SPMU287:      Stellaris Driver Installation Guide (for ICDI and FTDI drivers)

SPMU288:      BoosterPack Development Guide

SPMU289:     LaunchPad Evaluation Board User’s Manual (includes schematic)

You can find additional information at these websites:

       Main page: www.ti.com/launchpad

Stellaris LP: www.ti.com/stellaris-launchpad

EK-LM4F120XL product page: http://www.ti.com/tool/EK-LM4F120XL

BoosterPack webpage: www.ti.com/boosterpack

LaunchPad WiKi: www.ti.com/launchpadwiki

LM4F120H5QR  folder: http://www.ti.com/product/lm4f120h5qr

#### Kit Contents

- Open up your kit

You should find the following in your box:

  - The LM4F120H5QR LaunchPad Board

  - USB cable (A-male to micro-B-male)

  - README First card

  - If you are in a live workshop, you should find a 2nd USB cable

#### Initial Board Set-Up

- Connecting the board and installing the drivers

The LM4F120 LaunchPad Board ICDI USB port (marked DEBUG and shown in the picture below) is a composite USB port and consists of three connections:

Stellaris ICDI JTAG/SWD Interface 	- debugger connection
Stellaris ICDI DFU Device 			- firmware update connection

Stellaris Virtual Serial Port  		- a serial data connection

Using the included USB cable, connect the USB emulation connector on your evaluation board (marked DEBUG) to a free USB port on your PC. A PC’s USB port is capable of sourcing up to 500 mA for each attached device, which is sufficient for the evaluation board. If connecting the board through a USB hub, it must be a powered hub.

The drivers should install automatically. Manual driver installation steps are included in the appendix of this workbook.

#### QuickStart Application

Your LaunchPad Board came preprogrammed with a quickstart application. Once you have powered the board, this application runs automatically. You probably already noticed it running as you installed the drivers.

- Make sure that the power switch in the upper left hand corner of your board is in the right-hand DEBUG position as shown:

- The software on the LM4F120H5QR uses the timers as pulse-width modulators (PWMs) to vary the intensity of all three colors on the RGB LED (red, green, and blue) individually. By doing so, your eye will perceive many different colors created by combining those primary colors.

The two pushbuttons at the bottom of your board are marked SW1 (the left one) and SW2 (the right one). Press or press and hold SW1to move towards the red-end of the color spectrum. Press or press and hold SW2 to move towards the violet-end of the color spectrum.

If no button is pressed for 5 seconds, the software returns to automatically changing the color display.

- Press and hold both SW1 and SW2 for 3 seconds to enter hibernate mode. In this mode the last color will blink on the LEDs for ½ second every 3 seconds. Between the blinks, the device is in the VDD3ON hibernate mode with the real-time-clock (RTC) running. Pressing SW2 at any time will wake the device and return to automatically changing the color display.

- We can communicate with the board through the UART. The UART is connected as a virtual serial port through the emulator USB connection. 

The following steps will show how to open a connection to the board using HyperTerminal (in WinXP) and PuTTY (in Windows 7 or 8).

- We need to find the COM port number of the Stellaris Virtual Serial Port in the Device Manager. Skip to step 32 if you are using Windows 7 or 8.

Windows XP:

A. Click on the Windows Start button. Right-click on My Computer and select Properties from the drop-down menu.

B. In the System Properties window, click the Hardware tab.

C. Click the Device Manager button. 

The Device Manager window displays a list of hardware devices installed on your computer and allows you to set the properties for each device.

Expand the Ports heading and write number for the Stellaris Virtual Serial Port here: COM_____

- Windows 7 or 8:

- Click on the Windows Start button. Right-click on Computer and select Properties from the drop-down menu.

- Click on Device Manager on the left of the dialog.

The Device Manager window displays a list of hardware devices installed on your computer and allows you to set the properties for each device.

Expand the Ports heading and write number for the Stellaris Virtual Serial Port here: COM_____

- In WinXP, open HyperTerminal by clicking Start  Run…, then type hypertrm in the Open: box and click OK. Pick any name you like for your connection and click OK. In the next dialog box, change the Connect using: selection to COM##, where ## is the COM port number you noted earlier. Click OK.  Make the selections shown below and click OK.

When the terminal window opens, press Enter once and the LaunchPad board will respond with a > indicating that communication is open. Skip to step 31.

- In Win7 or 8, double-click on putty.exe. Make the settings shown below and then click Open. Your COM port number will be the one you noted earlier

When the terminal window opens, press Enter once and the LaunchPad board will respond with a > indicating that communication is open.

- You can communicate by typing the following commands and pressing enter:

help: will generate a list of commands and information

hib: will place the device into hibernation  mode. Pressing SW2 will wake the device.

rand: will start a pseudo-random sequence of colors

intensity: adjust the LED brightness between 0 to 100 percent. For instance     intensity 100 will change the LED to maximum brightness.

rgb: follow with a 6 hex character value to set the intensity of all three LEDs. For instance: rgb FF0000 lights the red LED, rgb 00FF00 lights the blue LED and rgb 0000FF lights the green LED.

- Close your terminal program.

You’re done.