# BoosterPacks and grLib

Introduction

This chapter will take a look at the currently available BoosterPacks for the LaunchPad board. We’ll take a closer look at the Kentec Display LCD TouchScreen BoosterPack and then dive into the StellarisWare graphics library.

## Chapter Topics

BoosterPacks and grLib	10-1

Chapter Topics	10-2

LaunchPad Boards and BoosterPacks	10-3

KenTec TouchSceen TFT LCD	10-7

Graphics Library	10-8

Lab 10: Graphics Library	10-11

Objective	10-11

Procedure	10-12

## LaunchPad Boards and BoosterPacks

See http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/default.aspx for a list of TI boosterpacks.

Solar Energy Harvesting: http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/archive/2011/06/08/cymbet-enerchip-cc-solar-energy-harvesting-evaluation-kit-cbc-eval-10.aspx

Universal Energy Harvesting:
http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/archive/2011/06/08/cymbet-enerchip-ep-universal-energy-harvesting-evaluation-kit-cbc-eval-09.aspx

Capacitive Touch:
http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/archive/2011/04/17/430boost_2d00_sense1.aspx

RF Module w/ LCD:
http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/archive/2011/07/13/golden-ic-rf-module-with-lcd-boosterpack.aspx

Inductive Charging:
http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/archive/2011/06/08/cymbet-enerchip-ep-universal-energy-harvesting-evaluation-kit-cbc-eval-11.aspx

Proto Board:
http://joesbytes.com/10-ti-msp430-launchpad-mini-proto-board.html

Olimex 8x8 LED Matrix:
http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/archive/2011/09/07/8x8-led-matrix-boosterpack-from-olimex.aspx

Sub-1GHz Wireless:
http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/archive/2011/12/01/texas-instruments-sub-1ghz-rf-wireless-boosterpack-430boost-cc110l.aspx

TPL0401 SPI Digital Potentiometer:
http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/archive/2011/11/18/texas-instruments-tpl0401-based-i2c-digital-potentiometer-tpl0401evm.aspx

TMP006 IR Temperature Sensor:
http://www.ti.com/tool/430boost-tmp006

C5000 Audio Capacitive Touch:
http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/archive/2012/03/27/texas-instruments-c5000-audio-capacitive-touch-boosterpack-430boost-c55audio1.aspx

TPL0501 SPI Digital Potentiometer:
http://e2e.ti.com/group/msp430launchpad/b/boosterpacks/archive/2011/11/18/texas-instruments-tpl0501-based-spi-digital-potentiometer-tpl0501evm.aspx

Proto Board:
http://store-ovhh2.mybigcommerce.com/ti-booster-packs/

LCD Controller Development Package:
http://www.epson.jp/device/semicon_e/product/lcd_controllers/index.htm

ZigBee Networking:
http://www.anaren.com/

MOD Board adapter:
https://www.olimex.com/dev/index.html

OLED Display:
http://www.kentecdisplay.com/plus/view.php?aid=74

Click Board Adapter:
http://www.mikroe.com/eng/categories/view/102/click-boards/

## KenTec TouchSceen TFT LCD

For more information go to: http://www.kentecdisplay.com/

## Graphics Library

This document: http://www.ti.com/lit/an/spma039/spma039.pdf has suggestions for modifying the display driver to connect to your display.

## Lab 10: Graphics Library

### Objective

In this lab you will connect the KenTec display to your LaunchPad board. You will experiment with the example code and then write a program using the graphics library.

### Procedure

#### Connect the KenTec Display to your LaunchPad Board

- Carefully connect the KenTec display to your LaunchPad board. Note the part numbers on the front of the LCD display. Those part numbers should be at the end of the LaunchPad board that has the two pushbuttons when oriented correctly. Make sure that all the BoosterPack pins are correctly engaged into the connectors on the bottom of the display.

#### Import Project

- We’re going to use the Kentec example project provided by the manufacturer. Maximize Code Composer and click Project  Import Existing CCS Eclipse Project. Make the settings shown below and click Finish. Note that this project will be automatically copied into your workspace.

- Expand the project in the Project Explorer pane, and then expand the drivers folder. The two files in this folder; Kentec320x240x16_ssd2119_8bit.c and touch.c are the driver files for the display and the touch overlay. Open the files and take a look around. Some of these files were derived from earlier StellarisWare examples, so you may see references to the DK-LM3S9B96 board.

Kentec320x240x16_ssd2119_8bit.c contains the low level Display Driver interface to the LCD hardware, including the pin mapping, contrast controls and simple graphics primitives.

#### Build, Download and Run the Demo

- Make sure your board is connected to your computer, and then click the Debug button to build and download the program to the LM4F120H5QR device. The project should build and link without any warnings or errors.

- Watch your LCD display and click the Resume button to run the demo program. Using the + and – buttons on-screen, navigate through the eight screens. Make sure to try out the checkboxes, push buttons, radio buttons and sliders. When you’re done experimenting, click Terminate on the CCS menu bar to return to the CCS Editing perspective.

#### Writing Our Own Code

- The first task that our lab software will do is to display an image. So we need to create an image in a format that the graphics library can understand. If you have not done so already, download GIMP from www.gimp.org and install it on your PC. The steps below will go through the process of clipping the photo below and displaying it on the LCD display. If you prefer to use an existing image or photograph, or one taken from your smartphone camera now, simply adapt the steps below.

- Make sure that this page of the workbook pdf is open for viewing and press PrtScn on your keyboard. This will copy the screen to your clipboard. The dimensions of the photo below approximate that of the 320x240 KenTec LCD.

- Open GIMP (make sure it is version 2.8 or later) and click Edit  Paste. In the toolbox window, click the Rectangle Select tool, and select tightly around the border of the photo. Zoom in if that is easier for you. Click Image  Crop to selection. Click Image  Scale Image and make sure that the image size width/height is 320x240 and click Scale. You may need to click the “chain” symbol to the right of the pixel boxes to stop GIMP from preserving the wrong dimensions.

- Convert the image to indexed mode by clicking Image  Mode  Indexed. Select Generate optimum palette and change the Maximum number of colors box to 16 (the color depth of the LCD). Click Convert.

- Save the file by clicking File  Export… Name the image pic, change the save folder to C:\StellarisWare\tools\bin and select PNM image as the file type using the 
+ Select File Type just above the Help button. Click Export. When prompted, select Raw as the data formatting and click Export. Close GIMP.

- Now that we have a source image file in PNM format, we can convert it to something that the graphics library can handle. We’ll use the pnmtoc (PNM to C array) conversion utility to do the translation.

Open a command prompt by clicking Start  Run. Type cmd in the window and click Open. The pnmtoc utility is in C:\StellarisWare\tools\bin. Type (Ctrl-V will not work)  cd C:\StellarisWare\tools\bin in the command window, then press Enter to change the folder to that location.

Finally, perform the conversion by typing pnmtoc –c pic.pnm > pic.c in the command window and hit Enter. When the process completes correctly, the cursor will simply drop to a new line. Close the DOS window.

- Using Windows Explorer, find the CCS workspace in your My Documents folder. Open the folder and find the grlib_demo folder that was copied here when you imported this project. Copy pic.c from C:\StellarisWare\tools\bin to the grlib_demo folder. 

Look back in the expanded grlib_demo project in the CCS Project Explorer. If the pic.c file does not appear there, right-click on the project and select Refresh.

#### Modify pic.c

- Open pic.c and add the following include to the very top of the file:

#include "grlib/grlib.h"

Your pic.c file should look something like this (your data will vary greatly):

#include "grlib/grlib.h"

const unsigned char g_pucImage[] =

{

IMAGE_FMT_4BPP_COMP,

96, 0,

64, 0,

15,

0x00, 0x02, 0x00,

0x18, 0x1a, 0x19,

0x28, 0x2a, 0x28,

0x38, 0x3a, 0x38,

0x44, 0x46, 0x44,

0x54, 0x57, 0x55,

0x62, 0x65, 0x63,

0x72, 0x75, 0x73,

0x81, 0x84, 0x82,

0x93, 0x96, 0x94,

0xa2, 0xa5, 0xa3,

0xb3, 0xb6, 0xb4,

0xc4, 0xc7, 0xc5,

0xd7, 0xda, 0xd8,

0xe8, 0xeb, 0xe9,

0xf4, 0xf8, 0xf5,

0xff, 0x07, 0x07, 0x07, 0x07, 0x07, 0x07, 0x07, 0x07, 0xff, 0x07, 0x07,

0x07, 0x07, 0x07, 0x07, 0x07, 0x07, 0xff, 0x07, 0x07, 0x07, 0x07, 0x07,

0x07, 0x07, 0x07, 0xfc, 0x07, 0x07, 0x07, 0x07, 0x07, 0x07, 0x03, 0x77,

0x23, 0x77, 0x77, 0xe9, 0x77, 0x78, 0x70, 0x07, 0x07, 0xc1, 0x77, 0x2c,

0x04, 0xde, 0xee, 0xee, 0xee, 0xe9, 0x3c, 0xee, 0xa1, 0x07, 0x07, 0x77,

0x2c, 0x03, 0xcf, 0x00, 0xee, 0xee, 0xee, 0xef, 0xee, 0xef, 0xfe, 0xa0,

0xf0, 0x07, 0x07, 0x77, 0x2c, 0x03, 0xcf, 0xee, 0xee, 0x4f, 0xee, 0xe9,

0xee, 0xa0, 0x07, 0x07, 0x77, 0x2c, 0x04, 0x03, 0xcf, 0xee, 0xee, 0xee,

0xe9, 0xee, 0x90, 0xf0, 0x07, 0x07, 0x77, 0x2c, 0x03, 0xcf, 0xee, 0xee,

0x4f, 0xee, 0xe9, 0xee, 0x90, 0x07, 0x07, 0x77, 0x2c, 0x04, 0x03, 0xcf,

many, many more lines of this data …

0x77, 0x2c, 0x19, 0xfe, 0xee, 0xef, 0x03, 0xee, 0xee, 0xee, 0xee, 0xfb,

0x20, 0x07, 0x07, 0xc1, 0x77, 0x2c, 0x05, 0xdf, 0xee, 0xee, 0xee, 0xe9,

0x78, 0xf9, 0x07, 0x07, 0x77, 0x2d, 0x01, 0x8d, 0xee, 0x2f, 0xee, 0xee,

0xe9, 0xf7, 0x07, 0x07, 0x77, 0x2e, 0x00, 0x39, 0xef, 0xee, 0xee, 0xee,

0xee, 0xee, 0xf7, 0xf0, 0x07, 0x07, 0x77, 0x2e, 0x06, 0xdf, 0xee, 0xee,

0x0f, 0xee, 0xee, 0xee, 0xf6, 0x07, 0x07, 0x77, 0x2f, 0x01, 0x7d, 0xfe,

0xee, 0xee, 0xee, 0xee, 0xf7, 0x07, 0xe0, 0x07, 0x77, 0x2f, 0x17, 0xdf,

0xee, 0xee, 0xee, 0x3c, 0xee, 0xf7, 0x07, 0x07, 0x77, 0x2f, 0x01, 0x7d,

0x03, 0xee, 0xee, 0xee, 0xee, 0xf9, 0x10, 0x07, 0x07, 0xc0, 0x77, 0x2f,

0x05, 0xad, 0xee, 0xfe, 0xee, 0xfc, 0x78, 0x20, 0x07, 0x07, 0x77, 0x2f,

0x00, 0x27, 0x9d, 0x0f, 0xed, 0xee, 0xec, 0x40, 0x07, 0x07, 0x77, 0x2f,

0x01, 0x00, 0x00, 0x28, 0x9a, 0xcc, 0xa9, 0x30, 0x07, 0xff, 0x07, 0x77,

0x2f, 0x07, 0x07, 0x07, 0x07, 0x07, 0xc0, 0x07, 0x07,

};

Save your changes and close the pic.c editor pane. If you’re having issues with this, you can find a pic.c file in the Lab10 folder.

#### Main.c

- To speed things up, we’re going to use the entire demo project as a template for our own main() code. But we can’t have grlib_demo.c in the project since it already has a main(). In the Project Explorer, right-click on grlib_demo.c and select Resource Configurations  Exclude from Build… Click the Select All button to select both the Debug and Release configurations, and then click OK. In this manner we can keep the old file in the project, but it will not be used during the build process. This is a valuable technique when you are building multiple versions of a system that shares much of the code between them.

- On the CCS menu bar, click File  New  Source File. Make the selections shown below and click Finish:

- Open main.c for editing. Add (or copy/paste) the following lines to the top:

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "driverlib/debug.h"

#include "driverlib/sysctl.h"

#include "grlib/grlib.h"
#include "drivers/Kentec320x240x16_ssd2119_8bit.h"

#### Pointer to the Image Array

- The declaration of the image array needs to be made, as well as the declaration of two variables. The variables defined below are used for initializing the Context and Rect structures. Context is a definition of the screen such as the clipping region, default color and font. Rect is a simple structure for drawing rectangles. Look up these APIs in the Graphics Library users guide .

Add a line for spacing and add the following lines after the includes:

extern const unsigned char g_pucImage[];

tContext sContext;
tRectangle sRect;

#### Driver Library Error Routine

- The following code will be called if the driver library encounters an error. 

Leave a line for spacing and enter these line of codes after the lines above:

#ifdef DEBUG
void__error__(char *pcFilename, unsigned long ulLine)
{
}
#endif

#### Main()

- The main() routine will be next. Leave a blank line for spacing and enter these lines of code after the lines above:

int main(void)

{

}

Initialization

- Set the clocking to run at 50 MHz using the PLL (400MHz ÷ 2 ÷ 4). Leave a line for spacing, then insert this line as the first inside main():

SysCtlClockSet(SYSCTL_SYSDIV_4 | SYSCTL_USE_PLL | SYSCTL_OSC_MAIN | SYSCTL_XTAL_16MHZ);

Initialize the display driver. Skip a line and insert this line after the last:

   	Kentec320x240x16_SSD2119Init();

This next function initializes a drawing context, preparing it for use. The provided display driver will be used for all subsequent graphics operations, and the default clipping region will be set to the extent of the LCD screen. Insert this line after the last:

GrContextInit(&sContext, &g_sKentec320x240x16_SSD2119);

- Let’s add a call to a function that will clear the screen. We’ll create that function in a moment. Add the following line after the last one:

ClrScreen();

- The following function will create a rectangle that covers the entire screen, set the foreground color to black, and fill the rectangle by passing the structure sRect by reference. The top left corner of the LCD display is the point (0,0) and the bottom right corner is (319,239).  Add the following code after the final closing brace of the program in main.c.

void ClrScreen()
{
   sRect.sXMin = 0;
   sRect.sYMin = 0;
   sRect.sXMax = 319;
   sRect.sYMax = 239;
   GrContextForegroundSet(&sContext, ClrBlack);
       GrRectFill(&sContext, &sRect);
   GrFlush(&sContext);
}

- Declare the function at the top of your code right below your variable definitions:

void ClrScreen(void);

#### Displaying the Image

- Display the image by passing the global image variable g_pucImage into GrImageDraw(...) and place the image on the screen by locating the top-left corner at (0,0) …we’ll adjust this later if needed. Leave a line for spacing, then insert this line after the ClrScreen() call in main():

GrImageDraw(&sContext, g_pucImage, 0, 0);

- The function call below flushes any cached drawing operations. For display drivers that draw into a local frame buffer before writing to the actual display, calling this function will cause the display to be updated to match the contents of the local frame buffer. Insert this line after the last:

GrFlush(&sContext);

- We will be stepping through a series of displays in this lab, so we want to leave each display on the screen long enough to see it before it is erased. The delay below will give you a chance to appreciate your work. Leave a line for spacing, then insert this line after the last:

SysCtlDelay(SysCtlClockGet());

In previous labs we’ve simply passed a number to the SysCtlDelay() API call, but if you were to change the CPU clock speed, your delay time would change. SysCtlClockGet() will return the system clock speed and we can use that as our delay basis. Obviously, you could have your delay be twice, half, 1/5th or some other multiple of this.

- Before we go any further, we’d like to take the code for a test run. With that in mind we’re going to add the final code pieces now, and insert later lab code in front of this.

LCD displays are not especially prone to burn in, but clearing the screen will mark a clear break between one step in the code and the next. This performs the same function as step 24 and also flushes the cache. Leave several lines for spacing and add this line below the last:

ClrScreen();

- Add a while loop to the end of the code to stop execution. Leave a line for spacing, then insert these line after the last:

while(1)

{

}

Don’t forget that you can auto-correct the indentation if needed.

If you’re having issues, you can find this code in main1.txt in the Lab10 folder.

Your code should look like this:

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "driverlib/debug.h"

#include "driverlib/sysctl.h"

#include "grlib/grlib.h"

#include "drivers/Kentec320x240x16_ssd2119_8bit.h"

extern const unsigned char g_pucImage[];

tContext sContext;

tRectangle sRect;

void ClrScreen(void);

#ifdef DEBUG

void__error__(char *pcFilename, unsigned long ulLine)

{

}

#endif

int main(void)

{

SysCtlClockSet(SYSCTL_SYSDIV_4|SYSCTL_USE_PLL|SYSCTL_OSC_MAIN|SYSCTL_XTAL_16MHZ);

Kentec320x240x16_SSD2119Init();

GrContextInit(&sContext, &g_sKentec320x240x16_SSD2119);

ClrScreen();

GrImageDraw(&sContext, g_pucImage, 0, 0);

GrFlush(&sContext);

SysCtlDelay(SysCtlClockGet());

// Later lab steps go between here

// and here

ClrScreen();

while(1)

{

}

}

void ClrScreen()

{

sRect.sXMin = 0;

sRect.sYMin = 0;

sRect.sXMax = 319;

sRect.sYMax = 239;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

GrFlush(&sContext);

}

#### Check the Build Options

- Now would be a good time to check the build options that have been set in this demo code. You should know how to do this without explicit steps by now. Take a look in the Linker’s File Search Path and note that the .lib file for the graphics library has been included. 

You might notice the use of two “new” path variables:

  - CG_TOOL_ROOT

  - SW_ROOT

Take at look in the project properties under Resource   Linked Resources to see where these paths are defined.

#### Run the Code

- Make sure grlib_demo is the active project. Compile and download your application by clicking the Debug button. Click the Resume button to run the program that was downloaded to the flash memory of your LM4F120H5QR. If your coding efforts were successful, you should see your image appear on the LCD display for a few seconds, then disappear.

When you’re finished, click the Terminate button to return to the CCS Edit perspective.

When you are including images in your projects, remember that they can be quite large in terms of memory space. This might possibly require a larger flash device, and increase your system cost.

#### Display Text On-Screen

- Refer back to the code on page 10-20. In main.c in the area marked:

// Later lab steps go between here

// and here

insert the following function call to clear the screen and flush the buffer:

ClrScreen();

- Next we’ll display the text. Display text starting at (x,y) with the no background color. The third parameter (-1) simply tells the API function to send the entire string, rather than having to count the characters.

GrContextForegroundSet(...): Set the foreground for the text to be red.

GrContextFontSet(...): Set the font to be a max height of 30 pixels.

GrRectDraw(...): Put a white border around the screen.

GrFlush(...): And refresh the screen by matching the contents of the local frame buffer.

Note the colors that are being used. If you’d like to try other colors, fonts or sizes, look in the back of the Graphics Library User’s Guide. Add the following lines after the previous ones:

   sRect.sXMin = 1;

sRect.sYMin = 1;

sRect.sXMax = 318;

sRect.sYMax = 238;

GrContextForegroundSet(&sContext, ClrRed);

GrContextFontSet(&sContext, &g_sFontCmss30b);

GrStringDraw(&sContext, "Texas", -1, 110, 2, 0);

GrStringDraw(&sContext, "Instruments", -1, 80, 32, 0);

GrStringDraw(&sContext, "Graphics", -1, 100, 62, 0);

GrStringDraw(&sContext, "Lab", -1, 135, 92, 0);

GrContextForegroundSet(&sContext, ClrWhite);

GrRectDraw(&sContext, &sRect);

GrFlush(&sContext);

- Add a delay so you can view your work.

SysCtlDelay(SysCtlClockGet());

Save your file.

If you’re having issues, you can find this code in main2.txt in the Lab10 folder.

Your added code should look like this:

// Later lab steps go between here

ClrScreen();

sRect.sXMin = 1;

sRect.sYMin = 1;

sRect.sXMax = 318;

sRect.sYMax = 238;

GrContextForegroundSet(&sContext, ClrRed);

GrContextFontSet(&sContext, &g_sFontCmss30b);

GrStringDraw(&sContext, "Texas", -1, 110, 2, 0);

GrStringDraw(&sContext, "Instruments", -1, 80, 32, 0);

GrStringDraw(&sContext, "Graphics", -1, 100, 62, 0);

GrStringDraw(&sContext, "Lab", -1, 135, 92, 0);

GrContextForegroundSet(&sContext, ClrWhite);

GrRectDraw(&sContext, &sRect);

GrFlush(&sContext);

SysCtlDelay(SysCtlClockGet());

#### // and here

#### Build, Load and Test

- Build, load and run your code. If your changes are correct, you should see the image again for a few seconds, followed by the on-screen text in a box for a few seconds. Then the display will blank out. Return to the CCS Edit perspective when you’re done.

#### Drawing Shapes

- Let’s add a filled-in yellow circle. Make the foreground yellow and center the circle at (80,182) with a radius of 50. Add a line for spacing and then add these lines after the SysCtlDelay() added in step 33:

GrContextForegroundSet(&sContext, ClrYellow);
GrCircleFill(&sContext, 80, 182, 50);

- Draw an empty green rectangle starting with the top left corner at (160,132) and finishing at the bottom right corner at (312,232). Add a line for spacing and add the following lines after the last ones:

sRect.sXMin = 160;
   sRect.sYMin = 132;
   sRect.sXMax = 312;
   sRect.sYMax = 232;
   GrContextForegroundSet(&sContext, ClrGreen);
   GrRectDraw(&sContext, &sRect);

- Add a short delay to appreciate your work. Add a line for spacing and add the following line after the last ones:

SysCtlDelay(SysCtlClockGet());

Save your work.

If you’re having issues, you can find this code in main3.txt in the Lab10 folder.

Your added code should look like this:

// Later lab steps go between here

ClrScreen();

sRect.sXMin = 1;

sRect.sYMin = 1;

sRect.sXMax = 318;

sRect.sYMax = 238;

GrContextForegroundSet(&sContext, ClrRed);

GrContextFontSet(&sContext, &g_sFontCmss30b);

GrStringDraw(&sContext, "Texas", -1, 110, 2, 0);

GrStringDraw(&sContext, "Instruments", -1, 80, 32, 0);

GrStringDraw(&sContext, "Graphics", -1, 100, 62, 0);

GrStringDraw(&sContext, "Lab", -1, 135, 92, 0);

GrContextForegroundSet(&sContext, ClrWhite);

GrRectDraw(&sContext, &sRect);

GrFlush(&sContext);

SysCtlDelay(SysCtlClockGet());

GrContextForegroundSet(&sContext, ClrYellow);

GrCircleFill(&sContext, 80, 182, 50);

sRect.sXMin = 160;

sRect.sYMin = 132;

sRect.sXMax = 312;

sRect.sYMax = 232;

GrContextForegroundSet(&sContext, ClrGreen);

GrRectDraw(&sContext, &sRect);

SysCtlDelay(SysCtlClockGet());

// and here

For reference, the final code should look like this:

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "driverlib/debug.h"

#include "driverlib/sysctl.h"

#include "grlib/grlib.h"

#include "drivers/Kentec320x240x16_ssd2119_8bit.h"

extern const unsigned char g_pucImage[];

tContext sContext;

tRectangle sRect;

void ClrScreen(void);

#ifdef DEBUG

void__error__(char *pcFilename, unsigned long ulLine)

{

}

#endif

int main(void)

{

SysCtlClockSet(SYSCTL_SYSDIV_4|SYSCTL_USE_PLL|SYSCTL_OSC_MAIN|SYSCTL_XTAL_16MHZ);

Kentec320x240x16_SSD2119Init();

GrContextInit(&sContext, &g_sKentec320x240x16_SSD2119);

ClrScreen();

GrImageDraw(&sContext, g_pucImage, 0, 0);

GrFlush(&sContext);

SysCtlDelay(SysCtlClockGet());

// Later lab steps go between here

ClrScreen();

sRect.sXMin = 1;

sRect.sYMin = 1;

sRect.sXMax = 318;

sRect.sYMax = 238;

GrContextForegroundSet(&sContext, ClrRed);

GrContextFontSet(&sContext, &g_sFontCmss30b);

GrStringDraw(&sContext, "Texas", -1, 110, 2, 0);

GrStringDraw(&sContext, "Instruments", -1, 80, 32, 0);

GrStringDraw(&sContext, "Graphics", -1, 100, 62, 0);

GrStringDraw(&sContext, "Lab", -1, 135, 92, 0);

GrContextForegroundSet(&sContext, ClrWhite);

GrRectDraw(&sContext, &sRect);

GrFlush(&sContext);

SysCtlDelay(SysCtlClockGet());

GrContextForegroundSet(&sContext, ClrYellow);

GrCircleFill(&sContext, 80, 182, 50);

sRect.sXMin = 160;

sRect.sYMin = 132;

sRect.sXMax = 312;

sRect.sYMax = 232;

GrContextForegroundSet(&sContext, ClrGreen);

GrRectDraw(&sContext, &sRect);

SysCtlDelay(SysCtlClockGet());

// and here

ClrScreen();

while(1)

{

}

}

void ClrScreen()

{

sRect.sXMin = 0;

sRect.sYMin = 0;

sRect.sXMax = 319;

sRect.sYMax = 239;

GrContextForegroundSet(&sContext, ClrBlack);

GrRectFill(&sContext, &sRect);

GrFlush(&sContext);

}

This is the code in main3.txt.

#### Build, Load and Test

- Build, load and run your code to make sure that your changes work. Return to the CCS Edit perspective when you are done.

#### Widgets

- Let’s play with some widgets. In this case, we’ll create a screen with a nice header and a large rectangular button that will toggle the red LED on and off. Modifying the existing code would be a little tedious, so we’ll create a new file.

- In the Project Explorer, right-click on main.c and select Resource Configurations  Exclude from Build… Click the Select All button to select both the Debug and Release configurations, and then click OK.

- On the CCS menu bar, click File  New  Source File. Make the selections shown below and click Finish:

- Add the following support files to the top of MyWidget.c:

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "driverlib/interrupt.h"

#include "driverlib/sysctl.h"

#include "driverlib/gpio.h"

#include "grlib/grlib.h"

#include "grlib/widget.h"

#include "grlib/canvas.h"

#include "grlib/pushbutton.h"

#include "drivers/Kentec320x240x16_ssd2119_8bit.h"

#include "drivers/touch.h"

- The next two lines provide names for structures needed to create the background canvas and the button widget. Add a line for spacing, then add these lines below the last:

extern tCanvasWidget g_sBackground;

extern tPushButtonWidget g_sPushBtn;

- When the button widget is pressed, a handler called OnButtonPress() will toggle the LED. Add a line for spacing, then add this prototype below the last:

void OnButtonPress(tWidget *pWidget);

- Widgets are arranged on the screen in order of a parent-child relationship, where the parent is in the back. This relationship can extend multiple levels. In our example, we’re going to have the background be the parent or root and the heading will be a child of the background. The button will be a child of the heading. Add a line for spacing and then add the following two global variables (one for the background and one for the button) below the last:

Canvas(g_sHeading, &g_sBackground, 0, &g_sPushBtn,

&g_sKentec320x240x16_SSD2119, 0, 0, 320, 23,

(CANVAS_STYLE_FILL | CANVAS_STYLE_OUTLINE | CANVAS_STYLE_TEXT),

ClrBlack, ClrWhite, ClrRed, g_pFontCm20, "LED Control", 0, 0);

Canvas(g_sBackground, WIDGET_ROOT, 0, &g_sHeading,

&g_sKentec320x240x16_SSD2119, 0, 23, 320, (240 - 23),

CANVAS_STYLE_FILL, ClrBlack, 0, 0, 0, 0, 0, 0);

Rather than re-print the parameter list for these declarations, refer to section 5.2.3.1 in the Stellaris Graphics Library User’s Guide (SW-GRL-UG-xxxx.pdf). The short description is that there will be a black background. In front of that is a white rectangle at the top of the screen with “LED Control” inside it.

- Next up is the definition for the rectangular button we’re going to use. The button is functionally in front of the heading, but physically located below it (refer to the picture in step 50). It will be a red rectangle with a gray background and “Toggle red LED” inside it. When pressed it will fill with white and the handler named OnButtonPress will be called. Add a line for spacing and then add the following code below the last:

RectangularButton(g_sPushBtn, &g_sHeading, 0, 0,

&g_sKentec320x240x16_SSD2119, 60, 60, 200, 40,

(PB_STYLE_OUTLINE | PB_STYLE_TEXT_OPAQUE | PB_STYLE_TEXT |

PB_STYLE_FILL), ClrGray, ClrWhite, ClrRed, ClrRed,

g_pFontCmss22b, "Toggle red LED", 0, 0, 0, 0, OnButtonPress);

Refer to section 10.2.3.33 in the Stellaris Graphics Library User’s Guide (spmu018n.pdf) for more detail.

- The last detail before the actual code is a flag variable to indicate whether the LED is on or off. Add a line for spacing and then add the following code below the last:

tBoolean g_RedLedOn = false;

- When the button is pressed, a handler called OnButton Press() will be called. This handler uses the flag to switch between turning the red LED on or off. Add a line for spacing and then add the following code below the last:

void OnButtonPress(tWidget *pWidget)

{

g_RedLedOn = !g_RedLedOn;

if(g_RedLedOn)

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1, 0x02);

}

else

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1, 0x00);

}

}

- Lastly is the main() routine. The steps are: initialize the clock, initialize the GPIO, initialize the display, initialize the touchscreen, enable the touchscreen callback so that the routine indicated in the button structure will be called when it is pressed, add the background and paint it to the screen (parents first, followed by the children) and finally, loop while the widget polls for a button press. Add a line for spacing and then add the following code below the last:

int main(void)

{

SysCtlClockSet(SYSCTL_SYSDIV_4|SYSCTL_USE_PLL|SYSCTL_OSC_MAIN|SYSCTL_XTAL_16MHZ);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3);

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, 0x00);

Kentec320x240x16_SSD2119Init();

TouchScreenInit();

TouchScreenCallbackSet(WidgetPointerMessage);

WidgetAdd(WIDGET_ROOT, (tWidget *)&g_sBackground);

WidgetPaint(WIDGET_ROOT);

while(1)

{

WidgetMessageQueueProcess();

}

}

Save your file.

If you’re having issues, you can find this code in MyWidget.txt in the Lab10 folder.

Your added code should look like the next page:

#include "inc/hw_memmap.h"

#include "inc/hw_types.h"

#include "driverlib/interrupt.h"

#include "driverlib/sysctl.h"

#include "driverlib/gpio.h"

#include "grlib/grlib.h"

#include "grlib/widget.h"

#include "grlib/canvas.h"

#include "grlib/pushbutton.h"

#include "drivers/Kentec320x240x16_ssd2119_8bit.h"

#include "drivers/touch.h"

extern tCanvasWidget g_sBackground;

extern tPushButtonWidget g_sPushBtn;

void OnButtonPress(tWidget *pWidget);

Canvas(g_sHeading, &g_sBackground, 0, &g_sPushBtn,

&g_sKentec320x240x16_SSD2119, 0, 0, 320, 23,

(CANVAS_STYLE_FILL | CANVAS_STYLE_OUTLINE | CANVAS_STYLE_TEXT),

ClrBlack, ClrWhite, ClrRed, g_pFontCm20, "LED Control", 0, 0);

Canvas(g_sBackground, WIDGET_ROOT, 0, &g_sHeading,

&g_sKentec320x240x16_SSD2119, 0, 23, 320, (240 - 23),

CANVAS_STYLE_FILL, ClrBlack, 0, 0, 0, 0, 0, 0);

RectangularButton(g_sPushBtn, &g_sHeading, 0, 0,

&g_sKentec320x240x16_SSD2119, 60, 60, 200, 40,

(PB_STYLE_OUTLINE | PB_STYLE_TEXT_OPAQUE | PB_STYLE_TEXT |

PB_STYLE_FILL), ClrGray, ClrWhite, ClrRed, ClrRed,

g_pFontCmss22b, "Toggle red LED", 0, 0, 0, 0, OnButtonPress);

tBoolean g_RedLedOn = false;

void OnButtonPress(tWidget *pWidget)

{

g_RedLedOn = !g_RedLedOn;

if(g_RedLedOn)

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1, 0x02);

}

else

{

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1, 0x00);

}

}

int main(void)

{

SysCtlClockSet(SYSCTL_SYSDIV_4|SYSCTL_USE_PLL|SYSCTL_OSC_MAIN|SYSCTL_XTAL_16MHZ);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOF);

GPIOPinTypeGPIOOutput(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3);

GPIOPinWrite(GPIO_PORTF_BASE, GPIO_PIN_1|GPIO_PIN_2|GPIO_PIN_3, 0x00);

Kentec320x240x16_SSD2119Init();

TouchScreenInit();

TouchScreenCallbackSet(WidgetPointerMessage);

WidgetAdd(WIDGET_ROOT, (tWidget *)&g_sBackground);

WidgetPaint(WIDGET_ROOT);

while(1)

{

WidgetMessageQueueProcess();

}

}

#### Build, Load and Test

- Build, load and run your code to make sure that everything works. Press the rectangular button and the red LED on the LaunchPad will light, press it again and it will turn off.

- Click the Terminate button to return to the CCS Edit perspective when you are done. Close all open lab projects and minimize Code Composer Studio.

- If you want to reprogram the qs-rgb application that was originally on the LaunchPad board, the steps are in section two of this workshop.

- Homework Ideas:

- Change the red background of the button so that it stays on when the LED is lit

- Add more buttons to control the green and blue LEDs.

- Use the Lab5 ADC code to display the measured temperature on the LCD in real time.

- Use the RTC to display the time of day on screen.

- Use the Lab6 Hibernation code to make the device sleep, and the backlight go off, after no screen touch for 10 seconds

- Use the Lab7 USB code to send data to the LCD and touch screen presses back to the PC.

- Use the Lab9 sine wave code to create a program that displays the sine wave data on the LCD screen.

You’re done.