[image: image16.wmf] 


 STYLEREF "ChapterTitle" \* MERGEFORMAT 

DaVinci AF Device Driver

[image: image16.wmf]
IMPORTANT NOTICE

Texas Instruments Incorporated and its subsidiaries (TI) reserve the right to make corrections, modifications, enhancements, improvements, and other changes to its products and services at any time and to discontinue any product or service without notice. Customers should obtain the latest relevant information before placing orders and should verify that such information is current and complete. All products are sold subject to TI’s terms and conditions of sale supplied at the time of order acknowledgment.

TI warrants performance of its hardware products to the specifications applicable at the time of sale in accordance with TI’s standard warranty. Testing and other quality control techniques are used to the extent TI deems necessary to support this warranty. Except where mandated by government requirements, testing of all parameters of each product is not necessarily performed.

TI assumes no liability for applications assistance or customer product design. Customers are responsible for their products and applications using TI components. To minimize the risks associated with customer products and applications, customers should provide adequate design and operating safeguards.

TI does not warrant or represent that any license, either express or implied, is granted under any TI patent right, copyright, mask work right, or other TI intellectual property right relating to any combination, machine, or process in which TI products or services are used. Information published by TI regarding third-party products or services does not constitute a license from TI to use such products or services or a warranty or endorsement thereof. Use of such information may require a license from a third party under the patents or other intellectual property of the third party, or a license from TI under the patents or other intellectual property of TI.

Reproduction of information in TI data books or data sheets is permissible only if reproduction is without alteration and is accompanied by all associated warranties, conditions, limitations, and notices. Reproduction of this information with alteration is an unfair and deceptive business practice. TI is not responsible or liable for such altered documentation.

Resale of TI products or services with statements different from or beyond the parameters stated by TI for that product or service voids all express and any implied warranties for the associated TI product or service and is an unfair and deceptive business practice. TI is not responsible or liable for any such statements.

Following are URLs where you can obtain information on other Texas Instruments products and application solutions:

	Products
	Applications

	Amplifiers
	amplifier.ti.com
	Audio
	www.ti.com/audio

	Data Converters
	dataconverter.ti.com
	Automotive
	www.ti.com/automotive

	DSP
	dsp.ti.com
	Broadband
	www.ti.com/broadband

	Interface
	interface.ti.com
	Digital Control
	www.ti.com/digitalcontrol

	Logic
	logic.ti.com
	Military
	www.ti.com/military

	Power Mgmt
	power.ti.com
	Optical Networking
	www.ti.com/opticalnetwork

	Microcontrollers
	microcontroller.ti.com
	Security
	www.ti.com/security

	
	
	Telephony
	www.ti.com/telephony

	
	
	Video & Imaging
	www.ti.com/video

	
	
	Wireless
	www.ti.com/wireless


Mailing Address:

Texas Instruments

Post Office Box 655303, Dallas, Texas 75265

Copyright © 2004, Texas Instruments Incorporated
Preface

Read This First


[image: image1.wmf]
About This Manual

This User’s Manual serves as a software programmer’s handbook for working with the DaVinci AF device driver. This manual provides necessary information regarding how to effectively install, build and use DaVinci AF device driver in user systems and applications. 

This manual provides details regarding how the DaVinci AF device driver is Architected, its composition, its functionality, the requirements it places on the hardware and software environment where it can be deployed, how to customize/ configure it to specific requirements, how to leverage the supported run-time interfaces in user’s own application etc.,

This manual also provides supplementary information regarding steps to be followed for proper installation/ un-installation of the DaVinci AF device driver. Also included are appendix sections on related Glossary, Web sites and Pointers for gathering further information on the DaVinci AF device driver. 

Terms and Abbreviations
	Term/Abbreviation
	Description

	DVEVM
	DaVinci Evaluation Module

	TII
	Texas Instruments India


Notations

NA

Information About Cautions and Warnings

NA
Related Documentation 

Internal

NA
External

· DM420_Spec_SOC_PRG_VPFE_B06.pdf 

Trademarks

The TI logo design is a trademark of Texas Instruments Incorporated. All other brand and product names may be trademarks of their respective companies.
This document contains proprietary information of Texas Instruments. The information contained herein is not to be used by or disclosed to third parties without the express written permission of an officer of Texas Instruments Incorporated.

Revision History 

	Date
	Author
	Revision History
	Version

	27/09/06
	Engineer 4
	Initial Draft Created
	0.01

	5/10/06
	Engineer 2
	Updated as per review 
	1.0

	6/3/07
	Engineer 4
	Removed AF Driver’s patch name from the document
	1.01


Contents


[image: image2.wmf]
iiiRead This First

About This Manual
iii
Terms and Abbreviations
iii
Notations
iii
Information About Cautions and Warnings
iii
Related Documentation
iv
Internal
iv
External
iv
Trademarks
iv
Revision History
iv
Contents
v
Tables
vi
Davinci AF Device Driver Introduction
0-1-1
1.1
Overview
0-1-2
1.1.1
Supported Services and features
0-1-2
1.1.2
System Requirements
0-1-2
Davinci AF Device Driver Design Overview
0-2-1
2.1
Modules
0-2-1
2.1.1
Initialization
0-2-1
2.1.2
Configuration & Control
0-2-1
2.1.3
Interrupt Handling
0-2-1
2.1.4
Buffer Management
0-2-2
2.1.5
Statistics
0-2-2
2.2
Layers
0-2-2
2.2.1
Functional Layer
0-2-2
2.2.2
HW Configuration Layer
0-2-2
Installation Guide
1-3-3
3.1
List of installable Components
1-3-4
3.2
Component Folder
1-3-5
3.3
Development Tools Environment(s)
1-3-6
3.4
Build
1-3-6
3.4.1
Build Options
1-3-6
3.4.2
Build Steps
1-3-6
3.5
Steps to Install DaVinci AF Device Driver
1-3-9
3.6
Steps to uninstall DaVinci AF Device Driver
1-3-9
Run-Time Interfaces/Integration Guide
2-4-10
4.1
Symbolic Constants and Enumerated Data types
2-4-11
4.2
Data Structures
2-4-13
4.3
API Classification
2-4-15
4.3.1
Configuration
2-4-16
4.3.2
Creation
2-4-16
4.3.3
Initialization
2-4-16
4.3.4
Control
2-4-16
4.3.5
Data Acquisition
2-4-16
4.3.6
Termination
2-4-17
4.4
API Usage Scenarios/Integration Example
2-4-17
4.5
API Specification
2-4-19
4.5.1
Naming Conventions
2-4-19
4.5.2
DaVinci AF Device Driver Functions
2-4-19
4.6
API Usage Recommendations
2-4-25
4.6.1
Achieving best performance
2-4-25
DaVinci AF device driver Resources
2-A-1


Tables


[image: image3.wmf]
1Table 1. Development Tools
-3-6
Table 2. Enumeration af_alaw_enable
2-4-11
Table 3. Enumeration config_flag
2-4-11
Table 4. Enumeration af_hmf_enable
2-4-11
Table 5. Enumeration rgbpos
2-4-11
Table 6. Enumeration af_mode
2-4-12
Table 7.Defines
2-4-12
Table 8.Structure af_paxel
2-4-14
Table 9.Structure af_hmf
2-4-14
Table 10.Structure af_iir
2-4-14
Table 11.Structure af_configuration
2-4-15
Table 16. API Close
2-4-19
Table 17. IOCTL AF_DISABLE
2-4-20
Table 18. IOCTL AF_ENABLE
2-4-20
Table 19. IOCTL AF_G_PARAM
2-4-21
Table 20. IOCTL AF_S_PARAM
2-4-22
Table 21. API Open
2-4-23
Table 22. API Read
2-4-24



1  SEQ Table \r 0 \h  \* MERGEFORMAT 

 SEQ Figure \r 0 \h    \* MERGEFORMAT 
Davinci AF Device Driver Introduction


[image: image4.wmf]
This chapter introduces the DaVinci AF device driver to the user by providing a brief overview of the purpose and construction of the DaVinci AF device driver along with hardware and software environment specifics in the context of DaVinci AF device driver deployment.

Topic
Page

	Overview
	1-2

	Supported Services
	1-2

	System Requirements
	1-2


1.1 Overview 

This section describes the functional scope of the DaVinci AF device driver and its feature set. The section also details the various deployment environments, hardware and software, that the DaVinci AF device driver is presently supported on. The chapter introduces the system architecture of the DaVinci AF device driver to the user along with the functional decomposition and run-time specifics regarding deployment of DaVinci AF device driver in user’s application.

A brief definition of the component is provided at this point – it’s main characteristics and purpose.

1.1.1 Supported Services and features

The DaVinci AF device driver provides the following functional services:

· AF Driver is Loadable Module

· AF Driver Supports image in bayer pattern.
· AF Driver supports input from CCDC.
1.1.2 System Requirements

The DaVinci AF device driver is supported on platforms characterized by the following Software and Hardware requirements.
1.1.2.1 Software

· Monta Vista Linux 2.6.10

1.1.2.2 Hardware

· DaVinci EVM Board.
2  SEQ Table \r 0 \h  \* MERGEFORMAT 

 SEQ Figure \r 0 \h    \* MERGEFORMAT 
Davinci AF Device Driver Design Overview


[image: image5.wmf]
This section provides the overview of the modules in DaVinci AF Device Driver. 
Topic
Page

	Modules
	2-1

	Initialization
	2-1

	Configuration
	2-1

	Interrupt handling
	2-1

	Buffer management
	2-2

	Layers
	2-2

	Functional Layer
	2-2

	HW Configuration Layer
	2-2


2.1 Modules

2.1.1 Initialization

This module is responsible to handle all the initialization activities including driver registration, driver un-registration, configuration creation and configuration deletion. 

2.1.2 Configuration & Control 

This module is responsible to handle AF functionality of the driver. 

2.1.3 Interrupt Handling
This is the interrupt handler for the AF driver. It will handle interrupt generated by H3A hardware for various events.
2.1.4 Buffer Management
This module is responsible for all buffer management activities including buffer creation and maintaining buffers.

2.1.5 Statistics

This module is responsible for providing latest statistics to the application. Read call behavior will be as follows.

· If the number of bytes requested by the application are less than the size of the available statistics then driver will return error to application. 

· If the number of bytes requested by the application are more than or equal to the size of available statistics data then driver will copy all the statistics data available in latest statistics buffer to the application.

· If the application issues read call and if new statistics are not available then driver will block the read call until new statistics are available or the timeout occurs. If new statistics are available before the timeout, driver will return new statistics to application, otherwise it will return error.

· Read Function should return error if it is already busy.
2.2 Layers

Complete AF driver will be divided in to two horizontal layers. 
2.2.1 Functional Layer
This layer will implement all the functionalities and application interface
2.2.2 HW Configuration Layer
This layer will contain functions to configure the HW. These functions will be used by the functional layer for configuration & control
.
Chapter 1  SEQ \h chapter \* MERGEFORMAT 
3  SEQ Table \r 0 \h  \* MERGEFORMAT 

 SEQ Figure \r 0 \h    \* MERGEFORMAT 
Installation Guide


[image: image6.wmf]
This chapter discusses the DaVinci AF Device Driver installation, how and what software and hardware components to be availed in order to complete a successful installation of DaVinci AF Device Driver.
Topic
Page

	List of installed Components
	3-2

	Component Folder
	3-3

	Development Tools Environment(s)
	3-4

	DaVinci AF  device driver Build
	3-4

	Build Options
	3-4

	Build Steps
	3-4

	Steps to install DaVinci AF device driver
	3-7

	Steps to un-install DaVinci AF  device driver
	3-7


3.1 List of installable Components

Patch containing DaVinci AF Device Driver code, Makefile and Kconfig files.
3.2 Component Folder

This section details the files and directory structure of the final installed DaVinci AF Device Driver in the system. 
[image: image7.jpg]B IrdavncLevmam, vt e
522 2610 mi401
€ Amo_versions
5 ach
& ewio

) Dosumentation
55 divers
2 acom
Q eoni
=
Q) base
& block
) bluetooth
& cdom
e ﬁ Em
Do
© dm
Q fape
=1
S oni





Figure 1. DaVinci AF Driver Directory Structure

3.3 Development Tools Environment(s)

This section describes the development tools environment(s) for software development with DaVinci AF Device Driver. It describes the tools used for each supported environment.

3.3.1.1 Development Tools 

We need to install following tools in given order to setup the development environment.

Table 1. Development Tools

	Development tool/ component
	Version
	Comments

	MVL401
	2.6.10
	

	MontaVista Linux Toolchain - arm_v5t_le-
	-
	

	
	
	


3.4 Build

3.4.1 Build Options

This driver does not have any specific build option at the time of writing of this manual.

3.4.2 Build Steps

Access to the AF Driver is provided through the /dev/davinci_af device file. The /dev/davinci_af device file is character devices that provide read/write access.
Following steps describes the procedure to enable DaVinci AF support in the system.

Step:1

Give ‘make ARCH=arm CROSS_COMPILE=arm_v5t_le- menuconfig’ command on kernel command prompt, which displays menu as below:

[image: image8.jpg]Arrow keys navigate the memu. <Enter> selects submenus

i

Highlighted letters are hotkeys. Pressing <¥> includes, <N> excludes,

<H> modularizes features. Press <Ese><Esc> to exit, <2> for Help

for Search. Legend: [] built-in [ ] excluded <H> module

<>

</>

ode maturity level options
eneral setup o
ocadshle module support
ystem type and features
De: -

N ntavista System tools
ernel hacking -
ecurity options -
ryptographic options
ibrary routines -





Select ‘Device Driver’ option.

Step 2:

Selection of ‘Device Drivers’ option displays menu as below.


[image: image9.jpg]Arrow keys navigate the menu. <Enter> selects submenus —-—>
Highlighted letters are hotkeys. Pressing <¥> includes, <N> excludes.
<i> modularizes features. Press <Ese><Esc> to exit, <2> for Help, </>
for Search. Legend: [¥] built—in [ ] excluded <N module < >

N tuorking support --->
TA/ATAPI/NFI/RLL support
CSI device support >
usion HPT device support
EEE 1394 (FireWire) support -
20 device support --->
SDN supsysten >
nput_device support

2C suppore —-—>

< Exit > < Help >





Select “Character Devices” option.

In the third step, DaVinci AF Driver can be selected for static build or as a Module. So Step 3 has two parts either Step 3.a or Step 3.b
Step 3.a:
Selection of “Character Devices” option displays menu as below.

[image: image10.jpg]Arrow keys navigate the menu. <Enter> selects submenus —-—>.
Highlighted letters are hotkeys. Pressing <¥> includes, <N> excludes,
<H> modularizes featurss. Press <Ese><Esc> to exit, <2> for Help, </>
for Search. Legend: [¥] built—in [ ] excluded <N> module < >

> / ev/mvram support

> nhanced Real Time Clock Support

> rioricy Inheritance Debugging (Blocker) Device Support

] T Davinei Real Time Clock

> ouble Talk PC internal speech card support

> iemens R3964 line diseipline

tape, the floppy tape device driver s

irect Rendering Nanager (XFreeSs 4.1.0 and higher DRI support)

AV driver [/dev/rau/raul) (OBSOLETE]

< Exit > < Help >






Select “DaVinci AF Driver Support” option as a static build.


This enables DaVinci AF Driver Support in kernel.


Now give ‘make ARCH=arm CROSS_COMPILE=arm_v5t_le- uImage’ command to build the kernel
Step 3.b:
[image: image11.jpg]Arrou keys navigate the memu. <Enter> selects submenus

5,

Highlighted letters are hotkeys. Pressing <¥> includes, <N> excludes,
<m>
for

modularizes features. Press <Esc><Ese> to exit, <?> for Help, </>
Search. Legend: [¥] built-in [ ] excluded <I> module < >

/ ev/nvran suppore

nhanced Real Time Clock Support

riority Inheritance Debugging (Blocker) Device Support

I Davinei Real Time Clock

ouwsle Talk PC internal speech card support

iemens R3964 line discipline

tape, the floppy tape device driver --->

irect Rendering Nanager (XFreess 4.1.0 and higher DRI support)
AU driver (/dev/ra/ravl) (OBSOLETE]

< Exit > < Help >





Select “DaVinci AF Driver Support” option as a module.


This enables DaVinci AF Driver Support in kernel as a module in the kernel.


Now give ‘make ARCH=arm CROSS_COMPILE=arm_v5t_le- modules’ command to build the modules.

3.5 Steps to Install DaVinci AF Device Driver
To install AF Driver:
· Copy AF driver’s patch to one directory level above the linux kernel directory

· From linux kernel directory give command “patch –p1 < ../AF Driver’s patch”
To install DaVinci AF Device Driver, write a command “insmod davinci_af_driver.ko”
3.6 Steps to uninstall DaVinci AF Device Driver

Driver can be removed from the kernel by “rmmod davinci_af_driver.ko” command.
Chapter 2  SEQ \h chapter \* MERGEFORMAT 
4  SEQ Table \r 0 \h  \* MERGEFORMAT 

 SEQ Figure \r 0 \h    \* MERGEFORMAT 
Run-Time Interfaces/Integration Guide


[image: image12.wmf]
This chapter discusses the DaVinci AF Device Driver run-time interfaces that comprise the API classification & usage scenarios and the API specification itself in association with its data types and structure definitions.

Topic 
Page
	Symbolic Constants and Enumerated Data types
	4-2

	Data Structures
	4-3

	API Classification
	4-7

	Configuration
	4-7

	Creation
	4-7

	Initialization
	4-7

	Control
	4-7

	Data acquisition/ Processing
	4-7

	Termination
	4-8

	API Usage Scenarios
	4-8

	API Specification
	4-11

	Naming Conventions
	4-11

	DaVinci AF Device Driver functions
	4-11

	API Usage Recommendations
	4-17

	Achieving best performance
	4-17


4.1 Symbolic Constants and Enumerated Data types

This section summarizes all the symbolic constants specified as either #define macros and/or enumerated C data types. Described alongside the macro or enumeration is the semantics or interpretation of the same in terms of what value it stands for and what it means.

Table 2. Enumeration af_alaw_enable
	Group or Enumeration Class
	Symbolic Constant Name
	Description or Evaluation

	af_alaw_enable
	H3A_AF_ENABLE = 1
	Enable Value.

	af_alaw_enable
	H3A_AF_DISABLE = 0 
	Disable Value The value is equal to 0.


Table 3. Enumeration config_flag
	Group or Enumeration Class
	Symbolic Constant Name
	Description or Evaluation

	af_config_flag
	H3A_AF_ENABLE = 1
	The value indicates that engine is configured

	af_config_flag
	H3A_AF_DISABLE = 0 
	The value indicates that engine is not configured.


Table 4. Enumeration af_hmf_enable

	Group or Enumeration Class
	Symbolic Constant Name
	Description or Evaluation

	af_hmf_enable
	H3A_AF_ENABLE = 1 
	Enable Value.

	af_hmf_enable
	H3A_AF_DISABLE = 0 
	Disable Value


Table 5. Enumeration rgbpos

	Group or Enumeration Class
	Symbolic Constant Name
	Description or Evaluation

	rgbpos
	GR_GB_BAYER = 0
	This value indicates GR and GB as Bayer pattern. 

	rgbpos
	RG_GB_BAYER = 1
	This value indicates RG and GB as Bayer pattern. 

	rgbpos
	GR_BG_BAYER = 2 
	This value indicates GR and BG as Bayer pattern. 

	rgbpos
	RG_BG_BAYER
	This value indicates RG and BG as Bayer pattern. 

	rgbpos
	GG_RB_CUSTOM
	This value indicates GR and RB as custom pattern. 

	rgbpos
	RB_GG_CUSTOM
	This value indicates RB and GG as custom pattern.


Table 6. Enumeration af_mode
	Group or Enumeration Class
	Symbolic Constant Name
	Description or Evaluation

	af_mode
	ACCUMULATOR_SUMMED = 0
	Summed  mode of the AF Accumulator.

	af_mode
	ACCUMULATOR_PEAK = 1
	Peak mode of  the AF Acuumulator.


Table 7.Defines
	Group or Enumeration Class
	Symbolic Constant Name
	Description or Evaluation

	Macro
	AF_ERR_HZ_COUNT
	Error code returned when user passes incorrect value of the horizontal count. The value of  the error code is 800.

	Macro
	AF_ERR_VT_COUNT
	Error code returned when user passes incorrect value of the horizontal count. The value of  the error code is 801.

	Macro
	AF_ERR_HEIGHT
	Error code returned when user passes incorrect value of height of the window. The value of  the error code is 802.

	Macro
	AF_ERR_WIDTH
	Error code returned when user passes incorrect value of height of the window. The value of  the error code is 803.

	Macro
	AF_ERR_INCR
	Error code returned when user passes incorrect value of increment. The value of  the error code is 804.

	Macro
	AF_ERR_HZ_START
	Error code returned when user passes incorrect value of horizontal start. The value of  the error code is 805.

	Macro
	AF_ERR_VT_START
	Error code returned when user passes incorrect value of vertical start. The value of  the error code is 806.

	Macro
	AF_ERR_IIRSH
	Error code returned when user passes incorrect value of height for black window. The value of  the error code is 807.

	Macro
	AF_ERR_IIR_COEF
	Error code returned when user passes incorrect value of vertical start for black window. The value of  the error code is 808.

	Macro
	AF_ERR_SETUP
	Error code returned when user tries to enable the engine without doing hardware configuration. The value of  the error code is 809.

	Macro
	AF_ERR_THRESHOLD
	Error code returned when user passes incorrect value of saturation limit. The value of  the error code is 810.

	Macro
	AF_ERR_ENGINE_BUSY
	Error code returned to indicate that engine is busy. The value of  the error code is 811.

	Macro
	AF_NUMBER_OF_COEF

	Error code returned when user passes incorrect value of width of the window. The number of coefficient is 11..

	Macro
	AF_MAGIC_NO
	Magic Number of AF The value of  magic no is ‘a’.

	Macro
	AF_PAXEL_SIZE
	Size of the paxel . The size of the paxel is 48.


4.2 Data Structures

This section summarizes all user visible data structures and their elements pertaining to the DaVinci AF Device Driver run-time interfaces.

 ‘af_paxel’ data structure is used to configure paxel parameters. The params are explained below:
Table 8.Structure af_paxel
	Group or Enumeration Class
	Symbolic Constant Name
	Description or Evaluation

	Unsigned int
	width
	Width of the window. The hardware will increment this value by one and then multiply it by 2. Range is 0 to 127.

	unsigned int
	height
	Height of the window. The hardware will increment this value by one and then multiply it by 2. Range is 0 to 127.

	unsigned int
	hz_start
	Horizontal Start of the paxel. Range is 2 to 4094.

	unsigned int
	vt_start
	Vertical Start of the paxel. Range is 0 to 4095.

	unsigned int
	hz_cnt
	Horizontal Count of the paxel. Hardware will increment this value by one. Range is 0 to 35.

	unsigned int
	vt_cnt
	Horizontal Count of the paxel. Hardware will increment this value by one. Range is 0 to 127.

	unsigned int
	line_incr
	Line Increment of the paxel. The hardware will increment this value by one and then multiply it by 2.Range is 0 to 14.


 ‘af_hmf’ data structure is used to configure HMF Filter parameters. The params are explained below:

Table 9.Structure af_hmf
	Group or Enumeration Class
	Symbolic Constant Name
	Description or Evaluation

	enum af_hmf_enable
	enable 
	Status of Horizontal Median Filter

	unsigned int
	threshold 
	Horizontal Filter Threshold. Maximum value is 255.


The file davinci_af.h has the ‘af_iir’ data structure that is used to configure IIR Filter parameters. The params are explained below:

Table 10.Structure af_iir
	Group or Enumeration Class
	Symbolic Constant Name
	Description or Evaluation

	Unsigned int
	hz_start_pos
	IIR Start Register Value. Range is 0 to 4094.

	Int
	coeff_set0[AF_NUMBER_OF_COEF];
	Coefficients of the IIR Filter 0. Each coefficient is 12 bits wide with 6 bits of the decimal precision. Coefficients are signed.

	Int
	coeff_set1[AF_NUMBER_OF_COEF]
	Coefficients of the IIR Filter 1. Each coefficient is 12 bits wide with 6 bits of the decimal precision. Coefficients are signed.


‘af_configuratiom’ data structure is used to configure AF Engine. The params are explained below:

Table 11.Structure af_configuration
	Group or Enumeration Class
	Symbolic Constant Name
	Description or Evaluation

	enum af_alaw_enable
	alaw_enable
	Enumeration value that is used to enable or disable alaw.

	enum rgbpos
	rgb_pos
	RGB Position.

	struct af_hmf
	hmf_config
	Variable of the type structure af_hmf. It store configuration for HMF Filter.

	struct af_iir
	iir_config
	Variable of the type structure af_iir. It store configuration for IIR Filters.

	struct af_paxel
	paxel_config;
	Variable of the type structure af_paxel. It store configuration for paxels.

	enum af_mode
	mode
	Accumulator mode

	enum af_config_flag
	af_config
	Enumeration value that indicates whether Engine is configured or not.


4.3 API Classification

This section introduces the application-programming interface for the DaVinci AF Device Driver by grouping them into logical units. .

4.3.1 Configuration

This section contains all the DaVinci AF Device Driver APIs that allow user to specify the desired configuration parameters. The IOCTL like AF_S_PARAMS helps the user to customize the DaVinci AF device driver parameters .The IOCTL Specifications section at the end will elaborate on each such mechanism in greater detail.

4.3.2 Creation

This section contains all the DaVinci AF Device Driver APIs that are intended for use in component creation. The term creation is indicative of possible need to allocate system resources, typically memory. 

The IOCTL like AF_S_PARAM will also dynamically allocate buffers as per the paxel count. It is mandatory for the user to configure all the parameters before enabling the engine. The IOCTL Specifications section at the end will elaborate on each such mechanism in greater detail.
4.3.3 Initialization

This contains the DaVinci AF Device Driver APIs that are intended for use in component initialization.

The API open is used for initializing of the AF configuration structure.
4.3.4 Control

This section contains DaVinci AF Device Driver APIs that is intended for controlling the functioning of DaVinci AF Device Driver during run-time. The IOCTL AF_ENABLE enables AF Engine. The IOCTL AF_DISABLE disables AF Engine. The IOCTL Specifications section at the end will elaborate on each such mechanism in greater detail.
4.3.5 Data Acquisition

This section should list all the DaVinci AF Device Driver APIs that help to output data out of DaVinci AF Device Driver. The IOCTL AF_G_PARAM helps user to get configuration parameters out of DaVinci AF Device Driver. The API read is used to get statistics. The behavior of Read API is as follows.
1) If the numbers of bytes requested by the application are less than the size of the available statistics then driver will return error to application.
2) If the numbers of bytes requested by the application are more than the size of available statistics data then driver will copy all the statistics data available in latest statistics buffer to the application.
3) If the application issues read call and if new statistics are not available then driver will block the read call until new statistics are available or the timer expires. If new statistics are available before the timer expires, driver will return new statistics to application.
4) Read Function should return error if it is already blocked.
4.3.6 Termination

This contains the DaVinci AF device driver APIs that help in gracefully terminating the deployed DaVinci AF device driver run-time entities.
The API close is used to free all the resources that are being acquired at time of Initialization and creation. 

4.4 API Usage Scenarios/Integration Example
Following two figures show the usage scenarios for DaVinci AF Device Driver. First figure shows simple single pass task. Second figure shows an example of multiple tasks submitted to the AF driver.

[image: image13.emf] 

Call open    

Call ioctl  A F _S_PARAM  

Call ioctl  A F _ ENBALE to enable  engine    

Call read call to get statistics    

Call ioctl AF_DISABLE to  disable engine    

Unload driver using rmmod    

Load the driver using insmod   

Call close    


 Figure 2: Function Flow Diagram

[image: image14.emf] 

Call open    

Call ioctl  A F _S_PARAM  

Call ioctl  A F _ ENBALE to enable  engine    

Call read call to get statistics    

Call ioctl AF_D ISABLE to  disable engine    

Unload driver using rmmod    

Load the driver using insmod   

Is it Last  Passs ?  

Call close    

No  

Yes  


Figure 3 Function Flow Diagram for multiple passes

4.5 API Specification

4.5.1 Naming Conventions
The naming conventions are followed as per the GA license.

4.5.2 DaVinci AF Device Driver Functions
The details description of the API discussed above are as follows in alphabetical order:-

1) Close

Table 16. API Close
	Prototype
	int close(int fd)

	Description
	Closes Device Driver opened with file descriptor 

	Arguments
	<arg1>
	int fd : File Descriptor

	
	<arg2>
	NA

	
	<arg3>
	NA

	Return value
	On success 0 is returned, on error -1 and the errno variable is set appropriately.

	Calling constraints
	None

	Example
	close(fd)

	Side effects
	None

	See Also
	None.

	Errors
	None


2) IOCTL AF_DISABLE

Table 17. IOCTL AF_DISABLE
	Prototype
	int ioctl(int fd, int command)

	Description
	This ioctl is used to disable AF Engine.

	Arguments
	<arg1>
	int fd : File Descriptor

	
	<arg2>
	int request

	
	<arg3>
	NA

	Return value
	On success 0 is returned, on error -1 and the errno variable is set appropriately.

	Calling constraints
	None.

	Example
	ioctl(fd,AF_DISABLE)

	Side effects
	None

	See Also
	None.

	Errors
	None.


3) IOCTL AF_ENABLE
Table 18. IOCTL AF_ENABLE
	Prototype
	int ioctl(int fd, int command)

	Description
	This ioctl is used to enable AF Engine with parameters set by AF_S_PARAM. 

	Arguments
	<arg1>
	int fd : File Descriptor

	
	<arg2>
	int request

	
	<arg3>
	NA

	Return value
	On success 0 is returned, on error -1 and the errno variable is set appropriately.

	Calling constraints
	None.

	Example
	ioctl(fd,AF_ENABLE)

	Side effects
	None

	See Also
	None.

	Errors
	AF_ERR_SETUP: If user tries to enable the engine without configuring the hardware


4) IOCTL AF_G_PARAM

Table 19. IOCTL AF_G_PARAM
	Prototype
	int ioctl(int fd, int command, struct af_configuration *arg)

	Description
	This ioctl is used to get the AF hardware settings.

	Arguments
	<arg1>
	int fd : File Descriptor

	
	<arg2>
	int request

	
	<arg3>
	struct af_configuration * config : pointer to configuration structure.

	Return value
	On success buffer size is returned, on error -1 and the errno variable is set appropriately.

	Calling constraints
	None.

	Example
	ioctl(fd,AF_G_PARAM,config)

	Side effects
	None

	See Also
	None.

	Errors
	EFAULT : Copy to user fails
AF_ERR_SETUP: If user tries to enable the engine without configuring the hardware


5) IOCTL AF_S_PARAM
Table 20. IOCTL AF_S_PARAM
	Prototype
	Int ioctl(int fd, int command, struct af_configuration *arg)

	Description
	This ioctl is used to set the following parameters/modules of the AF hardware: 

· Paxel parameters

· IIR Filter parameters

· HMF Filter Parameter
· A-Law compression module

	Arguments
	<arg1>
	int fd : File Descriptor

	
	<arg2>
	int request

	
	<arg3>
	struct af_configuration * config : pointer to configuration structure.

	Return value
	On success buffer size is returned, on error -1 and the errno variable is set appropriately.

	Calling constraints
	All the members into the af_configuration should be configured. There are no default values.

	Example
	ioctl(fd,AF_S_PARAM,config)

	Side effects
	None

	See Also
	None.

	Errors
	ENOMEM : If memory is not available

EFAULT : Copy from user fails

ENOTTY : If ioctl is incorrect

AF_ERR_ENGINE_BUSY :If engine is busy 

Driver Specific Error codes : If parameter are invalid

Refer to driver specific error codes in section 4.1


6) Open

Table 21. API Open

	Prototype
	int open(char *name, int mode)

	Description
	Opens the driver in the mode specified in the last parameter.

	Arguments
	Arg1
	char *name

	
	Arg2
	int mode

	
	Arg 3
	NA

	Return value
	File descriptor on success or -1 if an error occurred 

	Calling constraints
	None

	Example
	Open(“/dev/davinci_af”, O_RDWR)

	Side effects
	None

	See Also
	None

	Errors
	None


7) Read
Table 22. API Read
	Prototype
	ssize_t read(int fd, void *buf, size_t count)

	Description
	This API is used to get statistics from driver.
· If the numbers of bytes requested by the application are less than the size of the available statistics then driver will return error to application. 

· If the numbers of bytes requested by the application are more than the size of available statistics data then driver will copy all the statistics data available in latest statistics buffer to the application.

· If the application issues read call and if new statistics are not available then driver will block the read call until new statistics are available or the timer expires. If new statistics are available before the timer expires, driver will return new statistics to application.

· Read Function should return error if it is already blocked.

	Arguments
	<arg1>
	int fd : File Descriptor

	
	<arg2>
	void *buffer : buffer which is used to store statistics

	
	<arg3>
	size_t count : size of buffer 

	Return value
	Number of bytes read by the ‘read’ call.

	Calling constraints
	None.

	Example
	read(fd,buffer,count)

	Side effects
	None

	See Also
	None.

	Errors
	EBUSY: If read call is busy.

-1 : If count is less than size of buffer


4.6 API Usage Recommendations

This section provides recommendations on how to best use the provided APIs for achieving best results on different aspects – performance, overall system stability and balance, etc’.

4.6.1 Achieving best performance

Optimum performance can be achieved if line offsets are 256 bytes aligned. 

A  SEQ Table \r 0 \h  \* MERGEFORMAT 

 SEQ Figure \r 0 \h    \* MERGEFORMAT 
DaVinci AF device driver Resources


[image: image15.wmf]
This Appendix provides pointers to available resources to gather further information regarding DaVinci AF device driver. This helps the user to learn more about DaVinci AF device driver in general and seek information regarding specific capabilities and related tooling in order to better leverage the services extended by DaVinci AF device driver within his system or application.

The information keyed in here could include – online tutorials, multimedia demos, any built-in context sensitive help, published white papers or collateral documentation in form of technical briefs or application notes. Also listed will be applicable web-based resources both within TI and those offered by designated 3rd parties in support of DaVinci AF device driver.

· DaVinci AF device driver Information: www.dspvillage.com/DaVinci AFdevice driver
· Technical Documentation: www.ti.com/dsptechdocs/DaVinci AF device driver
· Knowledge Base: www.ti.com/dspkbase
· 3rd party catalog: www.ti.com/thirdpartycatalog
· Discussion Groups: www.ti.com/pspdiscussiongroups
· Online Training: www.ti.com/psptraining
· Workshops: www.ti.com/pspworkshops
· eStore: www.ti.com/pspestore
 SEQ Table \r 0 \h  \* MERGEFORMAT 

 SEQ Figure \r 0 \h    \* MERGEFORMAT 





















User’s Manual





� EMBED Word.Picture.8  ���




































































September 2006
Engineer 3

Document Version 1.0
iv
Texas Instruments Proprietary

This document contains information that is the confidential property of Texas Instruments Inc. It is provided under Nondisclosure and License and is not to be reproduced or distributed without prior written consent of Texas Instruments Inc.


[image: image17.wmf]User's Guide

[image: image18.wmf]User's Guide

_1220969034.doc

[image: image1]

Load the driver using insmod 























Call open	











Call close	







Unload driver using rmmod











Call ioctl AF_S_PARAM







Call ioctl AF_ENBALE to enable engine











Call read call to get statistics











Call ioctl AF_DISABLE to disable engine
















_1221565709.doc

[image: image1]

Load the driver using insmod 















Yes







Call close	







Call open	







No







Is it Last Passs?







Unload driver using rmmod











Call ioctl AF_S_PARAM







Call ioctl AF_ENBALE to enable engine











Call read call to get statistics











Call ioctl AF_DISABLE to disable engine
















_1049539699.doc



_1132476245.doc







_1049192503.doc
User's Guide



